Nuclei® Xunlian Subsystem Quick-start Manual

Nucleisys

All rights reserved by Nucleisys, Inc.

Contents

1    Copyright Notice

2    Contact Information

3    Nuclei SoC Subsystem Delivery Package

3. 1     File Package Hierarchy

3.2     View RTL Code

4    System Simulation

4.1     C/C++ Test Case

4.1.1      The user added Ctest

4.2     Simulation Platform

4.3     Run C Test simulation

4.4     Run SRAM Scan

4.5     Connect JTAG GDB

4.5.1     Integrate JTAGVPI into your env

5   FPGA Validation

5. 1     Generate FPGA Version

5.2     Run C Test Use Case

5.3     USART Functional Testing

5.3.1     Introduction

5.3.2      Running results

5.4     SPI Functional Testing

5.4.1     Introduction

5.4.2      Running results

5.4.3      Read and Write Flash Test

5.5     TIMER Functional Testing

5.5.1      Test Program Introduction

5.5.2      Running Results

5.6     TIMER_pwmout Functional Testing

5.6.1      Test Program Introduction

5.6.2      Running Results

5.7     I2C Eeprom-AT24C02 Testing

5.7.1      Test Program Introduction

5.8     SDIO Functional Testing

5.8.1     Introduction

5.8.2      Running Results

5.9     Ethernet Functional Testing

5.9.1     Introduction

5.9.2      Running Results

5. 10   USB Functional Testing

5.10.1   Introduction

3

4

5

5

6

7

7

8

8

8

10

10

13

14

14

15

16

16

16

17

17

17

17

18

18

18

18

18

19

19

19

19

20

20

20

20

20

21

21

5.10.2    Running Results    	    21

5. 11   CAN Functional Testing     	    22

5.11.1   Introduction    	    22

5.11.2    Running Results    	    22

6    DC Synthesis                                                                                                                                                       23

6.1     Synthesis Steps    	    23

7    Caution                                                                                                                                                                24

8    Appendix                                                                                                                                                             25

List of Figures

4. 1    Printf simulation output results     	    10

4.2    Waiting for JTAG connection     	    11

4.3    JTAG connection information    	    12

4.4    JTAG connection successful       	    12

4.5    GDB Start     	    12

4.6    GDB debugging helloworld program     	    13

5. 1    Helloworld sample program C test run results       	    16

5.2    USART operation results      	    16

5.3    Test results      	    17

5.4    Read and write flash test results     	    18

5.5    TIMER pwmout test results    	    19

5.6    I2C Eeprom-AT24C02I2C test results      	    19

5.7    SDIO running results      	    20

5.8    Ethernet running results    	    21

5.9    USB running results    	    21

5. 10  CAN running results       	    22

List of Tables

3. 1    The Package Introduction .   .   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .        5

# 1 Copyright Notice

Copyright◎ 2018-2026 Nuclei System Technology. All rights reserved.

Nuclei®, Nucleisys®, NMSIS®, ECLIC®, are trademarks owned by Nuclei System Technology.  All other trademarks used herein are the property of their respective owners.

The product described herein is subject to continuous development and improvement; information herein is given by Nuclei in good faith but without warranties.

This document is intended only to assist the reader in the use of the product.  Nuclei do not assume any liability arising out of the application or use of any product or circuit, and specifically disclaims any and all liability, including without limitation indirect, incidental, special, exemplary, or consequential damages. incorrect use of the product.

# 2 Contact Information

Should you have any problems with the information contained herein or any suggestions, please contact Nuclei System Technology by email support@nucleisys.com, or visit “Nuclei User Center” website http://user.nucleisys.comfor supports or online discussion.

# 3 Nuclei SoC Subsystem Delivery Package

The content of the delivered by the Nuclei SoC Subsystem, xunlian_rls_pkg.tar.gz is a file compressed package, the intro- duction is shown in The Package Introduction (page 5) .

The following is the version of EDA tool we use:

synopsys/dc-2021.06-SP1

synopsys/vcs-2022.06-SP2-1  synopsys/verdi-2022.06-SP2-1 fpga/vitis-2021.1

python/python-3.7.3

Table 3.1: The Package Introduction

| Package Name | xunlian_*.tar.gz |
| --- | --- |
| Introduction | Including Testbench for the Verilog RTL delivery,SoC,Simulation Environment and FPGA source code file compression package. |

The files of Nuclei SoC Subsystem be obtained from Nuclei. Decompress the xunlian_*.tar.gz folder in the Linux system.

# Decompress  the  xunlian_rls_pkg  to  the  native  Linux  environment .

#  See  the  readme  file  of the  FTP path  for  details .

$  unzip  private_* .zip

# Enter password

$  tar  -zxvf  xunlian.tar.gz

$  cp  private .pem  ./xunlian_rls_pkg/

$  cd  xunlian_rls_pkg

$  source  env.sh (env .csh)

$  ./nuclei_soc_gen

## 3.1  File Package Hierarchy

The files in the xunlian_rls_pkg.tar.gz package are introduced as below(subject to the actual delivery ip).

xunlian_rls_pkg

L- xunlian

rtl                                       #  Store  rtl  code  for RISC-V SoC  designs

l   -  core                            #  CPU rtl  code

l   -  main_fab                   #  Store  the  rtl  code  for  the  SoC  bus  fabric

l   -  usart                           #  Store  the  rtl  code  for  the  usart

l   -  <ip_name>                    #  Store  the  rtl  code  for  the  <ip_name>

(continues on next page)

(continued from previous page)

...

doc                                       # Reference  Manual

addr_map .csv             #  Memory  map  and  register  boundary

Nuclei_xunlian_Subsystem_Quick_Start_Manual.pdf Nuclei_xunlian_Reference_Manual.pdf

<cpu>-doc-<version> .zip  #  CPU Reference  Manual

...

# Main  Makefile

#  Store  synthetical  netlist  information

#  Information  generated  when  running  tests

# Directory  for  Verilog  Testbench

#  Storage  of JTAG related  file

#  Storage  of flashmodel  related  files

#  Simple  Verilog  TestBench  file .

# Directory  where  test  cases  are  stored

#  Storage  of ctest  test  cases

# Directory  where  FPGA projects  and  scripts  are  stored

#  Store  the  SoC  corresponding  driver  and  Ctest  test

...

tests

L- core0

L- system_test fpga

nuclei-sdk

Makefile syn

test_run

testbench

jtag_vpi

flash_model tb_* .v

addresses

...

Note

RTL contains a lot of RTL source code, which contains the processor core and all the synthesizable RTL source code of SoC. Users only need to care about the IP in the subsystem, and do not need to care about the IP without it.

## 3.2 View RTL Code

Suppose the user wants to quickly view and compile the source code of the Nuclei SoC Subsystem, you can use the following steps to proceed.

#  Step  1:  Go  to  the  xunlian_rls_pkg/xunlian  directory.

$  cd  xunlian_rls_pkg/xunlian

#  Step  2:  View  RTL  code .

$ make  verdi_rtl

# 4 System Simulation

Nuclei SoC Subsystem provides software development kits (SDK, Software Development Kit). For details, please refer to Nuclei-SDK User Guide.

## 4.1  C/C++ Test Case

The C/C++ test cases are some demo programs developed by Nuclei, which need to be carried out with the help of nuclei- sdk. The relevant C program files are stored in the following directory(subject to the actual delivery ip).

xunlian_rls_pkg xunlian

nuclei-sdk

soc_test_cases

ns                        #  Store  the  test  case  directory  of the  soc

USART              #  USART  test  program

I2C                #  I2C  test program

WWDG                  #  WWDG  test  program

LGPIO              # LGPIO  test  program

SPI                  #  SPI  test program

ADV_TIMER      #  ADV_TIMER  test  program

BSC_TIMER      #  BSC_TIMER  test  program

top_ctest     #  top  test program

. . .

These test programs are written in C/C++ language. Under nuclei-sdk/soc_test_cases/ns , include script files, C/C++ and header files. If the compiled file does not exist, the user needs to compile the C/C++ program by himself. The following is an example of Printf test case of usart0. The compiled program is an elf file and a disassembly file, as well as a file that can be read by the readmemh function of Verilog, as shown below.

. . .

main .c

Makefile

README.TXT

USART_usart0_printf.elf

USART_usart0_printf.dump

#  C  source  File

#  test  README  file

#  Compiled  elf file

# Disassembly  file

USART_usart0_printf.verilog  #  can  be  read  by  the  $readmemh

USART

L- usart0

L- printf

# Printf

#  usart0

test

test

case

case

Note

The *.elf, *.dump, *.verilog files are generated only after the test program is executed.

### 4.1.1 The user added Ctest

The new cases follow the following directory structure:

1.

nuclei-sdk/soc_test_cases/ns/<IP_NAME>/<ip_instance_name>/<case_name>/main.c

nuclei-sdk/soc_test_cases/ns/<IP_NAME>/<ip_instance_name>/<case_name>/Makefile

Ctest name is <IP_NAME>_<ip_instance_name>_<case_name> 2.

nuclei-sdk/soc_test_cases/ns/top_ctest/<ctest_name>/main.c

nuclei-sdk/soc_test_cases/ns/top_ctest/<ctest_name>/Makefile Ctest name is <ctest_name>

Ctests for both of these rules can be compiled using ctest_gun How to use ctest_gun see :$ ctest_gun –help

## 4.2 Simulation Platform

A simple Testbench written in Verilog has been created in the xunlian_rls_pkg/xunlian directory.

| xunlian_rls_pkg L- xunlian L- testbench | # The Verilog TestBench(Test platform) |
| --- | --- |
| — tb_* .v | # Simple Verilog TestBench file . |
| ... |  |

The main functions in the testbed are as follows:

•  Routine DUT file, generate clock and reset signals

•  The name of the test case is resolved according to the run command, and the contents of the corresponding file (e.g. USART_usart0_printf.verilog) are read in using Verilog’s readmemh function, and then the contents of the file are used to initialize the SoC’s Instruction Menory (a two-dimensional array written in Verilog that acts as the SRAM behavior model).

Note

Users can also integrate correlation tb_*.v into the SoC to facilitate running self-test cases in the SoC environment.

## 4.3  Run C Test simulation

Assuming that a program written in C/C++ is run in the user simulation environment, this can be done using the following steps.  Using the sample program(Printf) as an example, you can proceed using the following steps.  The SDK, which requires users to contact Nuclei to obtain.

Note

If there is already a *.verilog file under the provided tests/core0/system_tests path, it does not need to be compiled by the user.

Step 1:Compile c test programs.

$  cd  xunlian_rls_pkg/xunlian

$  chmod  +x  testbench/script/ctest_gun

#  ilm  Compile  c  test  steps:

$  ctest_gun  -t  USART_usart0_printf  --bm=ilm

#  sramxip  Compile  c  test  steps:

$  ctest_gun  -t  USART_usart0_printf  --bm=sramxip

#  flash  Compile  c  test  steps:

$  ctest_gun  -t  USART_usart0_printf  --bm=flash<0/1/2>

#  flashxip  Compile  c  test  steps:

$  ctest_gun  -t  USART_usart0_printf  --bm=flashxip<0/1/2>

Step 2:Run the C test executable under the simulation platform.

$  cd  xunlian_rls_pkg/xunlian

#  ilm  Start procedure  steps:

$ make  run  TESTNAME=USART_usart0_printf_ilm

#  sramxip  Start procedure  steps:

$ make  run  TESTNAME=USART_usart0_printf_sramxip

#  flash  Start procedure  steps:

$ make  run  TESTNAME=USART_usart0_printf_flash<0/1/2>

#  flashxip  Start procedure  steps:

$ make  run  TESTNAME=USART_usart0_printf_flashxip<0/1/2>

# make  run  will  compile  the  rtl  code  and  then  execute  the  Printf sample  program  under

#  the path .

#  ilm Restart procedure  steps:

$ make  ncrun  TESTNAME=USART_usart0_printf_ilm

#  sramxip  Restart procedure  steps:

$ make  ncrun  TESTNAME=USART_usart0_printf_sramxip

#  flash  Restart procedure  steps:

$ make  ncrun  TESTNAME=USART_usart0_printf_flash<0/1/2>

#  flashxip  Restart procedure  steps:

$ make  ncrun  TESTNAME=USART_usart0_printf_flashxip<0/1/2>

# make  ncrun  will  directly  execute  the  Printf sample program  under  the path .

Note

make run is equivalent to make compile and make ncrun.  When switching ctest, you do not need to recompile rtl. Please use make ncrun to save time

Step 3:Run wave(View Waveform),use the following command.

#  ilm  Call  verdi  steps:

$ make  verdi  TESTNAME=USART_usart0_printf_ilm

#  sramxip  Call  verdi  steps:

$ make  verdi  TESTNAME=USART_usart0_printf_sramxip<0/1/2>

#  flash  Call  verdi  steps:

$ make  verdi  TESTNAME=USART_usart0_printf_flash<0/1/2>

#  flashxip  Call  verdi  steps:

$ make  verdi  TESTNAME=USART_usart0_printf_flashxip<0/1/2>

#  Call  verdi  to  see  the  waveform  after  this  test  is  executed.

# Module  core_wrapper  under  Trace  and  other related  interfaces  are  used  to  output  the

#  internal  status  information  required  by  external  devices,  which  can  debug  with  this

#  information .

C Test simulation results (Printf for example) as shown in Printf simulation output results (page 10) .

Fig. 4.1: Printf simulation output results

## 4.4  Run SRAM Scan

The Verilog Testbench source codes are under the “testbench” directory as below.

xunlian_rls_pkg L- xunlian

testbench

— tb_sram_scan .v    #  sram  scan  source  codes ...

The SRAM Scan module is used to test all SRAM of the Core/Cluster, if user has replace the SRAM, then can integrate this module to the SoC and run the task to check the SRAM replacement is complete before running software.

$  ctest_gun  -t  tb_sram_scan  --bm=<ilm  or  sramxip> $ make  run  TESTNAME=tb_sram_scan_<ilm  or  sramxip>

When it shows PASS, it is all OK. When it shows FAIL, user should check more details.

Note

It should not run any other case when do sram scan task

## 4.5  Connect JTAG GDB

Use the following steps to emulate the JTAG debug connection, using openocd in the emulation environment to confirm the JTAG debug debug interface connection.

Step 1:Compile c test programs.

$  cd  xunlian_rls_pkg/xunlian

#  ilm  Compile  c  test  steps:

$  ctest_gun  -t  USART_usart0_printf  --bm=ilm

#  sramxip  Compile  c  test  steps:

$  ctest_gun  -t  USART_usart0_printf  --bm=sramxip

#  flash  Compile  c  test  steps:

$  ctest_gun  -t  USART_usart0_printf  --bm=flash

(continues on next page)

(continued from previous page)

#  flashxip  Compile  c  test  steps:

$  ctest_gun  -t  USART_usart0_printf  --bm=flashxip

Step 2:Compile the RTL, Run the compiled C/C++ program, using the following command:

$  cd  xunlian_rls_pkg/xunlian

# Go  to  the  xunlian_rls_pkg/xunlian  directory

#  ilm  Start procedure  steps:

$ make  run  TESTNAME=USART_usart0_printf_ilm  JTAGVPI=1

#  sramxip  Start procedure  steps:

$ make  run  TESTNAME=USART_usart0_printf_sramxip  JTAGVPI=1

#  flash  Start procedure  steps:

$ make  run  TESTNAME=USART_usart0_printf_flash<0/1/2>  JTAGVPI=1

#  flashxip  Start procedure  steps:

$ make  run  TESTNAME=USART_usart0_printf_flashxip<0/1/2>  JTAGVPI=1

#  ilm Restart procedure  steps:

$ make  ncrun  TESTNAME=USART_usart0_printf_ilm  JTAGVPI=1

#  sramxip  Restart procedure  steps:

$ make  ncrun  TESTNAME=USART_usart0_printf_sramxip  JTAGVPI=1

#  flash  Restart procedure  steps:

$ make  ncrun  TESTNAME=USART_usart0_printf_flash<0/1/2>  JTAGVPI=1

#  flashxip  Restart procedure  steps:

$ make  ncrun  TESTNAME=USART_usart0_printf_flashxip<0/1/2>  JTAGVPI=1 #  JTAGVPI=1  is  using  JTAG  debugging  and  waiting  for  JTAG  connection .

The simulation is correctly waiting for the JTAG connection results as shown in Waiting for JTAG connection (page 11) .

Fig. 4.2: Waiting for JTAG connection

Step 3:Start openocd emulation JTAG connection

Download from Nuclei Toolchain <https://www.nucleisys.com/download.php>

$  tar  zxvf nuclei-openocd-2022 .04-linux-x64 .tgz

$  cd  xunlian_rls_pkg/xunlian/nuclei-sdk/SoC/ns/Board/fpga_eval

$  ${your_path}/Nuclei/openocd/2022.04/bin/openocd  -f  openocd_ns_sim.cfg

Note

This step needs to be executed immediately after the second make run step to avoid the openocd connection timeout.

Using openocd simulation JTAG connection correct results as shown in JTAG connection information (page 12) .

Fig. 4.3: JTAG connection information

After successful connection, the JTAG VPI connection can be seen in the waiting connection simulation interface:

Fig. 4.4: JTAG connection successful

Step 4:Start GDB emulation debugging:

$  cd  soc_test_cases/ns/USART/usart0/printf

# Go  to  the  corresponding  test  directory,  here  is  an  example  of USART_usart0_printf. $  make  run_gdb

# Perform  GDB  debugging.

GDB starts as shown in GDB Start(page 12) . GDB debugging results (take USART_usart0_printf as an example) are as GDB debugging helloworld program (page 13) .

Fig. 4.5: GDB Start

Fig. 4.6: GDB debugging helloworld program

### 4.5.1  Integrate JTAGVPI into your env

If you want to integrate the JTAGVPI into your env to test the jtag connect function, you need belows files.

• jtag_vpi.c

• jtag_vpi.tab

• jtag_vpi.v

Note

these files are in testbench/jtag_vpi dir in your soc package.

Step 1:add below args into your compile arg.

-P  jtag_vpi.tab  -CC  "-DVCS_VPI"  jtag_vpi.c  jtag_vpi.v

Step 2:instance the jtagvpi module in your testbench, and connect the jtag signal.

bit  jtag_enable  =  1 'b0 ;

JTAGVPI  u_jtagvpi(

.jtag_TMS      (jtag_tms),

.jtag_TCK      (jtag_tck),

.jtag_TDI     (jtag_tdi),

.jtag_TDO      (jtag_tdo),

.jtag_TRST    (1 'b0),  // unused

.enable          (1 'b1),

.init_done    (jtag_enable) );

initial begin

// use  the  simulation  arg  to  control  the  enable  for jtag  vpi  test

if($test$plusargs("JTAG_VPI"))begin jtag_enable  =  1 'b1 ;

end

end

Step 3:add below args into your simulation arg.

+JTAG_VPI  +jtag_port=6666

•  +JTAG_VPI : enable the jtag vpi test

•  +jtag_port: specify the jtag use port

# 5 FPGA Validation

Please refer to the FPGA hardware manual for details of the RISC-V customized dedicated JTAG debugger and dedicated FPGA development board.

Note

Users should pay attention to the size of the SRAM, if you need to increase or decrease your own SRAM size, you can adjust according to the RAM depth.  Nuclei in order to adapt to its own spontaneous version, maybe reduce the RAM depth.

If the program is running in SRAM, you need to change the size of the link script SRAM to the corresponding size.

$  vi  nuclei-sdk/SoC/ns/Board/fpga_eval/Source/GCC/gcc_ns_sram .ld #  sram  (rxa!w)       :  ORIGIN  =  0x1c000000,  LENGTH  =  256K

## 5.1 Generate FPGA Version

The FPGA version can be generated using the following steps.

#  To  generate  the  FPGA  version,  use  the  following  command:

$  cd  xunlian_rls_pkg/xunlian/fpga/

# Go  to  the  xunlian_rls_pkg/xunlian/fpga  directory $ module  load  fpga/vitis-2020 .1

# First,  the  user  should  install  vivado  and  click  set  to  environment  variable  during installation .  (For  example,  load  vivado  command: module  load  FPGA  /  vitis-2020 . 1) If  it  has  been  installed,  this  sentence  can  be  ignored .

$  make mcs

#  Compile  RTL:if expand("%")  ==  ""|browse  confirm  w|else|confirm  w|endif

## 5.2  Run C Test Use Case

The test cases for chip FPGA verification are programs written in C/C++ language and can be performed using the following steps. It needs to be carried out with the help of nuclei-sdk. Take the sample program (helloworld) as an example, you can use the following steps to proceed.

#  Step  1:To  compile  the  helloworld  sample  program,  use  the  following  command:

$  cd  soc_test_cases/ns/USART/usart0/printf

#  The  ctestcase path  of the  test $ make  clean

$  make  dasm  BOARD=fpga_eval  DOWNLOAD=ilm

# DOWNLOAD=ilm    Download  as  ilm

Note

If option DOWNLOAD=ilm is specified here, the program will be compiled by “downloading the program to ILM for execution”.

#  Step  2:Download  the  compiled  C  test  program  into  the  FPGA  prototype  development  board

#  using  the  following  command:

$  make  upload    BOARD=fpga_eval  DOWNLOAD=ilm

#  Step  3:Running  the  helloworld program  on  the  FPGA  prototype  development  board:

# As  the  sample program  will  need  to print  the  structure  to  the  host  PC's

#  display  via  USART.  Refer  to  the  method  described  in  Section  Error:  Reference  source  not

#  found  to  set  up  the  serial  port  to  display  the  computer  screen  so  that  the  program 's

# print  information  can  be  displayed  on  the  computer  screen .

Note

MISA varies depending on the customer’s choice of RISC-V core.

Fig. 5.1: Helloworld sample program C test run results

# 5.3  USART Functional Testing

If your Nuclei SoC Subsystem has USART, please refer to the following instructions:

### 5.3.1  Introduction

The test program is used to verify the serial communication functions and interrupts, and its functions are briefly described as follows:

•  Configure USART0 baud rate to 115200, data bit to 8bit, stop bit to 1bit, disable hardware flow control.

•  Enables global interrupts and USART0 transmit and receive interrupts.

•  The USART sends a specific string through the middle and the upper bit receives it through the serial debug assistant. Wait for the serial debug assistant of the upper unit to send the string to the USART.

•  The upper unit sends characters to the USART (the number of characters is greater than 32) and outputs the test result “successfully” through the printf function.

### 5.3.2  Running results

The results of the test run on the chip FPGA verification board are shown in USART operation results (page 16) :  Serial debug assistant sends data to FPGA verification board (number of characters > 32).

Fig. 5.2: USART operation results

## 5.4 SPI Functional Testing

If your Nuclei SoC Subsystem has SPI, please refer to the following instructions:

### 5.4.1  Introduction

The program can be downloaded in flashxip mode to verify the communication function of SPI, and its function is briefly described as follows:

•  Set the DOWNLOAD parameter to flashxip.  SPI0 and SPI1 send and receive  10 bytes of data to and from each other. Finally, verify that the data sent and received are the same, as reflected by the LED2 and LED3.

### 5.4.2  Running results

Download the test program to run on the FPGA verification board and observe LED2 and LED3.  LED2 and LED3 are lit, which means that SPI0 and SPI1 send and receive data consistently and the communication function is nor- mal.(LED2:Green Light;LED3:Blue Light)

Fig. 5.3: Test results

### 5.4.3  Read and Write Flash Test

•  SPI is initially in Master mode

•  Read flash ID and print

•  Erase 3byte address mode Flash

•  Write 3byte address mode Flash

•  Read 3byte address mode Flash

•  Calibration data

Fig. 5.4: Read and write flash test results

## 5.5 TIMER Functional Testing

If your Nuclei SoC Subsystem has TIMER, please refer to the following instructions:

### 5.5.1 Test Program Introduction

TIMER_timebase test program is used to verify the Timer timing function, which is briefly described as follows:

•  Configure Timer1 divider, count value, count mode and auto reload, and channel CH0 set to output compare mode.

•  Enable CH0 interrupt, when the count value is equal to the threshold, generate an interrupt and print in the interrupt Headler.

•  TIMER0 periodically generates interrupts and the serial port also performs periodic printing.

### 5.5.2  Running Results

Download the test program to run on the chip FPGA verification board and observe the serial port printing. The serial port prints periodically and Timer0 timing function is normal.

## 5.6 TIMER_pwmout Functional Testing

If your Nuclei SoC Subsystem has TIMER_pwmout, please refer to the following instructions:

### 5.6.1 Test Program Introduction

•  Configure Timer divider, count value, count mode and auto reload. Channels CH1, CH2 and CH3 are set to PWM mode.

•  Set the duty cycle of CH1 to 25%, CH2 to 50%, and CH3 to 75%.

### 5.6.2  Running Results

Download the test program to run on the chip FPGA verification board. The output results are shown in TIMER pwmout test results (page 19) , and the waveform results are normal.

Fig. 5.5: TIMER pwmout test results

## 5.7  I2C Eeprom-AT24C02 Testing

If your Nuclei SoC Subsystem has I2C Eeprom-AT24C02, please refer to the following instructions:

### 5.7.1 Test Program Introduction

The test program uses I2C to read and write Eeprom(AT24C02). The functions are briefly described as follows:

•  Set I2C0 to Master mode

•  Enable I2C0 transmit and receive, configure Eeprom’s Slave address

•  Initialize data, write Eeprom

•  Read back data for verification

•  Output results

Fig. 5.6: I2C Eeprom-AT24C02I2C test results

## 5.8 SDIO Functional Testing

If your Nuclei SoC Subsystem has SDIO, please refer to the following instructions:

### 5.8.1  Introduction

The test program is used to verify the SDIO function of reading and writing SD cards, and its function is briefly described as follows:

•  Send cmd through SDIO to identify the type of SD card

•  Get the CID, CSD, etc. of the SD card and parse it out

•  Read and write 512 bytes (one block size) of data

•  Compare the written and read data, if the data is the same, then print successfully and display the SD card information, otherwise print fail

### 5.8.2  Running Results

Firstly, insert the SD card into the card slot. Download the test program. The result of running the test program on the chip FPGA verification board.  Observe the serial debug assistant printing, which indicates that the test read/write is normal and the card information is correctly parsed.

Fig. 5.7: SDIO running results

## 5.9  Ethernet Functional Testing

If your Nuclei SoC Subsystem has Ethernet, please refer to the following instructions:

### 5.9.1  Introduction

The test program is used to verify the RGMII communication function of the Ethernet interface, and its functions are briefly described as follows:

•  Ported lwip protocol line, the program set the development board IP ADDR: 192.168.200.10 ,UDP Port:8089

•  After using the network debug assistant to bind the UDP port number, send any data to the development board, and the board will ECHO back the same data.

### 5.9.2  Running Results

First of all, the IP address of the upper unit is set up, and the IP address of the development board is in the same network segment, and the ETH1 network port of the development board is connected to the upper unit through the network cable. After downloading the program, the development board will first send the “Nuclei XEC UDP demo send data” string to the PC, which means the PC and the development board network connection is normal. Then the PC will send any data to the development board, and the development board will ECHO back the same data.

Fig. 5.8: Ethernet running results

## 5.10  USB Functional Testing

If your Nuclei SoC Subsystem has USB, please refer to the following instructions:

### 5.10.1  Introduction

The test program is used to verify the communication function of the USB interface, and its functions are briefly described as follows:

•  Use ULPI interface to connect to external USB phy, development board connects to PC via type-c interface.

•  Enumeration experiments using the interface descriptors of Audio devices.

### 5.10.2  Running Results

Download the driver for USB to the development board.  Then plug the development board USB port to PC. The PC’s device manager will refresh an USB Audio Device.

# Fig. 5.9: USB running results

## 5.11  CAN Functional Testing

If your Nuclei SoC Subsystem has CAN, please refer to the following instructions:

### 5.11.1  Introduction

The test program is used to verify function CAN loopback. Its function is briefly described as follows:

•  Configure the CAN to loopback mode, use the CAN to send several bytes of data, and read the received data. Compare sending and receiving data.

•  Enables the CAN receive interrupt.  Use CAN to send two bytes of data.  And read the received data.  Consistently print success.

### 5.11.2  Running Results

Fig. 5.10: CAN running results

# 6

# DC Synthesis

This SoC package(xunlian_*.tar.gz) provides comprehensive Flow-related makefile, tcl scripts and constraint files.

## 6.1 Synthesis Steps

After configuring the relevant parameters and lib path in syn/Makefile, you can run DC synthesis with the following command.

$  cd  xunlian_rls_pkg/xunlian/syn/

$  make  dc

Note

The above files need to be modified according to the user’s EDA tool configuration.

# 7 Caution

Please pay attention to the following notes.

Note

Users should pay attention to the base addr(0x1c000000, no device region) of test_sram, if you need to use the test_fab slv port, you can not use the address range(0x1c000000~0x1c0fff)for your device.

# 8 Appendix

• Nuclei RISC-V IP Products: https://www.nucleisys.com/product.php

•  Nuclei Spec Documentation: https://nucleisys.com/download.php#spec

•  Nuclei RISCV Tools and Documents: https://nucleisys.com/download.php

•  Nuclei Prebuilt Toolchain and IDE:https://nucleisys.com/download.php#tools

• NMSIS:https://github.com/Nuclei-Software/NMSIS

• Nuclei SDK:https://github.com/Nuclei-Software/nuclei-sdk

• Nuclei Linux SDK:https://github.com/Nuclei-Software/nuclei-linux-sdk

•  Nuclei Software Organization in Github: https://github.com/Nuclei-Software/

• RISC-V MCU Organization in Github: https://github.com/riscv-mcu/

• RISC-V MCU Community Website: https://www.rvmcu.com/

•  Nuclei riscv-openocd: https://github.com/riscv-mcu/riscv-openocd

•  Nuclei riscv-gnu-toolchain: https://github.com/riscv-mcu/riscv-gnu-toolchain